# SOP_008: CRISPR Library Validation

Standard Operating Procedure for sgRNA Representation Analysis | isabel.quintanilla@nih.gov, gianluca.pegoraro@nih.gov

Isabel Quintanilla

January 13, 2026

## 1 Purpose and Scope

### 1.1 Objective

To validate CRISPR pooled cell libraries through genomic DNA extraction, PCR amplification, gel purification, next-generation sequencing, and computational analysis using MAGeCK pipeline to assess sgRNA representation and library quality.

### 1.2 Scope

This protocol applies to CRISPR pooled cell libraries requiring validation before Optical Pooled Screening (OPS) and covers the complete workflow from cell harvesting through bioinformatics analysis of sequencing data.

### 1.3 Prerequisites

Knowledge of CRISPR library design and sgRNA representation requirements

Basic cell culture training

PCR and gel electrophoresis experience

Access to Biowulf computational cluster

Familiarity with command-line operations

## 2 Materials and Equipment

### 2.1 Equipment Required

Centrifuge for cell pellets

Thermocycler (PCR machine)

Electrophoresis apparatus and power supply

UV transilluminator or blue light system

Qubit fluorometer or equivalent

TapeStation (optional)

Nanodrop spectrophotometer

Freezer (-20°C and -80°C)

### 2.2 Reagents and Consumables

| Reagent/Item | Supplier | Catalog # | Storage | Notes |
| --- | --- | --- | --- | --- |
| Qiagen DNeasy Blood & Tissue Kit | Qiagen | 69504 | RT | For gDNA extraction |
| Q5 High-Fidelity 2X Master Mix | NEB | M0492S | -20°C | For PCR amplification |
| P5 primer (10 μM) | IDT | Custom | -20°C | Forward primer |
| P7 primer (10 μM) | IDT | Custom | -20°C | Reverse primer |
| Agarose | Various | - | RT | For gel electrophoresis |
| GelRed (10,000X) | Biotium | 41003 | 4°C | Nucleic acid stain |
| 1X TAE/TBE buffer | Various | - | RT | Electrophoresis buffer |
| Qiagen Gel Extraction Kit | Qiagen | 28704 | RT | For gel purification |
| Qubit dsDNA HS Assay Kit | ThermoFisher | Q32851 | 4°C | DNA quantification |
| Nuclease-free water | Various | - | RT | PCR reactions |

### 2.3 Software Requirements

MAGeCK software (on Biowulf cluster)

Terminal/command-line access

File transfer software (for data download)

Spreadsheet software for data analysis

## 3 Reagent Preparation

### 3.1 2% Agarose Gel

Final Volume: 120 mL Storage: Use fresh or store at 4°C for up to 1 week

| Component | Amount | Notes |
| --- | --- | --- |
| Agarose | 1.0 g | High resolution grade |
| 1X TAE buffer | 120 mL | Warm to dissolve agarose |
| GelRed | 5 μL | Add after cooling to ~60°C |

Preparation Steps:

Dissolve agarose in TAE buffer by microwaving in 30-second intervals

Cool to approximately 60°C

Add GelRed and swirl gently

Pour into gel casting tray and insert comb

### 3.2 PCR Master Mix (per 50 μL reaction)

Components for single reaction:

| Component | Volume | Final Concentration |
| --- | --- | --- |
| Q5 High-Fidelity 2X Master Mix | 25 μL | 1X |
| P5 primer (10 μM) | 2.5 μL | 0.5 μM |
| P7 primer (10 μM) | 2.5 μL | 0.5 μM |
| Template DNA | Variable | ~1.5 μg |
| Nuclease-free water | To 50 μL | - |

## 4 Procedure

### 4.1 Cell Harvesting and gDNA Extraction

Time Required: 2 hours Temperature: Room temperature

Cell Harvesting Harvest sufficient cells to maintain proper sgRNA representation Aim for minimum 500 cells per sgRNA in library ⚠️ Critical Point: Maintain library representation throughout process

Cell Pelleting Centrifuge cells according to standard protocols Remove supernatant completely Freeze cell pellet immediately at -80°C if not proceeding immediately

gDNA Extraction Use Qiagen DNeasy Blood & Tissue kit following manufacturer’s instructions ⚠️ Critical Point: Maximum 5×10⁶ cells per extraction For larger libraries, perform multiple extractions Elute DNA in nuclease-free water (not elution buffer)

DNA Quantification Measure DNA concentration using Nanodrop ✅ Quality Check: A260/A280 ratio should be 1.8-2.0 Calculate total DNA yield

### 4.2 PCR Amplification

Time Required: 3-4 hours Temperature: Thermocycling

PCR Setup Use approximately 1.5 μg template DNA per 50 μL reaction Set up multiple reactions for large libraries Include negative control (no template)

Thermocycling Program

| Step | Temperature | Time | Cycles |
| --- | --- | --- | --- |
| Initial Denaturation | 98°C | 3 min 30 sec | 1 |
| Denaturation | 98°C | 30 sec | 30 |
| Annealing | 53°C | 30 sec | 30 |
| Extension | 72°C | 45 sec | 30 |
| Final Extension | 72°C | 2 min | 1 |
| Hold | 12°C | ∞ | - |

PCR Product Pooling Mix all PCR reactions from same sample Store at -20°C if not proceeding immediately

### 4.3 Gel Electrophoresis and Purification

Time Required: 2-3 hours Temperature: Room temperature

Gel Electrophoresis Load portion of pooled PCR product on 2% agarose gel Use multiple lanes if necessary Run at appropriate voltage (typically 100-120V) ✅ Quality Check: Expected band size ~300 bp

Band Excision Visualize DNA under UV or blue light Excise gel band containing ~300 bp fragment ⚠️ Critical Point: Work quickly to minimize UV exposure

Gel Purification Use Qiagen Gel Extraction kit following manufacturer’s instructions Pool all purified DNA from same sample Elute in minimal volume of elution buffer

Final Quantification Quantify purified DNA using Qubit dsDNA HS Assay Optional: Analyze on TapeStation for size verification ✅ Quality Check: Confirm expected fragment size and concentration

### 4.4 Next-Generation Sequencing

Time Required: 1-2 weeks (external service)

Sample Submission Submit purified PCR products to Genomics facility Request NextSeq 2000 XLEAP P1 100 cycle sequencing Specify ~100M reads requirement Provide sample information and barcode details

## 5 Bioinformatics Analysis

### 5.1 Data Download and Preparation

Platform: Biowulf computational cluster Time Required: 2-4 hours

Interactive Session Setup sinteractive

Data Download wget "[sequencing_facility_link]" -O sequencing_data.tar ⚠️ Critical Point: Use exact URL provided by sequencing facility

File Extraction tar -xf sequencing_data.tar cd OUTPUT/Data/fastq gunzip * .fastq.gz

Reference Library Preparation Copy reference library file to working directory Format reference file using dos2unix if necessary: dos2unix reference_library.txt

### 5.2 MAGeCK Analysis Pipeline

Time Required: 1-2 hours

Load MAGeCK Module module load MAGeCK

Read Counting mageck count \ -l reference_library.txt \ -n sample_name \ --sample-label "T0" \ --fastq sample_R1_001.fastq

Quality Control Analysis Review MAGeCK count output files Check read mapping statistics Assess sgRNA representation

### 5.3 Data Interpretation

Key Metrics to Evaluate Total read count and mapping efficiency sgRNA representation (minimum 500 reads per sgRNA recommended) Gini coefficient (library evenness) Percentage of sgRNAs with zero counts

Quality Thresholds Mapping efficiency: >80% sgRNA coverage: >95% of library Gini coefficient: <0.1 for well-balanced libraries

## 6 Quality Control and Validation

### 6.1 Critical Control Points

CCP 1: Maintain minimum 500 cells per sgRNA during harvesting

CCP 2: DNA purity (A260/A280 ratio 1.8-2.0)

CCP 3: PCR product size verification (~300 bp)

CCP 4: Successful gel purification with appropriate yield

CCP 5: Sequencing read mapping efficiency >80%

### 6.2 Expected Results

Clean PCR product at expected size (~300 bp)

High-quality sequencing data with good mapping rates

Even distribution of sgRNA representation

Minimal dropout of library elements

### 6.3 Acceptance Criteria

Pass: >95% sgRNA coverage, Gini coefficient <0.1, mapping efficiency >80%

Conditional Pass: 90-95% coverage, may proceed with noted limitations

Fail: <90% coverage or mapping efficiency <70%, repeat library preparation

## 7 Troubleshooting

| Problem | Possible Cause | Solution | Prevention |
| --- | --- | --- | --- |
| Low DNA yield | Insufficient cells, poor extraction | Increase cell number, fresh reagents | Optimize cell harvest timing |
| Poor PCR amplification | Degraded DNA, primer issues | Check DNA quality, verify primers | Store DNA properly, validate primers |
| Multiple bands on gel | Non-specific amplification | Optimize annealing temperature | Use high-fidelity polymerase |
| Low mapping efficiency | Contamination, wrong reference | Check reference file format | Verify library design |
| Uneven sgRNA distribution | PCR bias, library construction issues | Reduce PCR cycles, check original library | Optimize PCR conditions |
| High Gini coefficient | Library bias, clonal expansion | Assess original library quality | Monitor culture conditions |

## 8 References and Related Documents

### 8.1 Related SOPs

SOP_009a: Conventional OPS Library Preparation

SOP_009b: T7 Library Preparation

SOP_010: SBS Illumina Sequencing

SOP_011: Imaging Using Nikon Ti2

### 8.2 Literature References

Li, W. et al. MAGeCK enables robust identification of essential genes from genome-scale CRISPR/Cas9 knockout screens. Genome Biol 15, 554 (2014).

Doench, J.G. et al. Optimized sgRNA design to maximize activity and minimize off-target effects of CRISPR-Cas9. Nat Biotechnol 34, 184-191 (2016).

### 8.3 Web Resources

MAGeCK documentation: https://sourceforge.net/p/mageck/wiki/Home/

Biowulf MAGeCK guide: https://hpc.nih.gov/apps/MAGeCK.html

## 9 Appendices

### 9.1 File Naming Conventions

Raw sequencing data: [sample_name]_S[number]_R1_001.fastq

Reference library: [library_name]_3cols.txt (tab-delimited: sgRNA_ID, sequence, gene)

MAGeCK output: [sample_name].count.txt , [sample_name].countsummary.txt

### 9.2 Biowulf Specific Notes

Work in /data/username directory for optimal performance

Use sinteractive for interactive sessions

Load modules before running analysis tools

Transfer large files using appropriate methods

### 9.3 Expected File Formats

Reference Library Format (tab-delimited):