# SOP_009a: Conventional OPS Library Preparation

Standard Operating Procedure for Optical Pooled Screening | isabel.quintanilla@nih.gov, gianluca.pegoraro@nih.gov

Isabel Quintanilla

January 13, 2026

## 1 Purpose and Scope

### 1.1 Objective

To prepare cell samples for Optical Pooled Screening (OPS) using conventional reverse transcription, padlock probe hybridization, and rolling circle amplification to enable in situ sequencing of cellular transcripts.

### 1.2 Scope

This protocol applies to cultured cells grown to 70% confluence and covers the complete workflow from sample fixation through rolling circle amplification. Phenotyping steps, are not included in this protocol, but can be performed either after postfixation or after the Phi29/RCA amplification.

### 1.3 Prerequisites

Cells cultured to 70% confluence for 48 hours

Basic cell culture training

RNase-free technique training

Knowledge of fluorescence microscopy (if performing phenotyping)

## 2 Materials and Equipment

### 2.1 Equipment Required

30-45°C incubator

Flat PCR machine/thermocycler

Fluorescent microscope

Glass bottom plates (6-well plates, CellVis #P06-1.5H-N)

Pipettes and RNase-free tips

Fume hood

### 2.2 Reagents and Consumables

| Reagent Name | Supplier | Catalog # | Storage | Final Concentration | Price ($) |
| --- | --- | --- | --- | --- | --- |
| Paraformaldehyde 32% (10x10mL) | Electron Microscopy Sciences | 15714 | 4°C | 4% (fresh) | 50.00 |
| PBS 10X RNase-free (1L) | ThermoFisher Scientific | AM9625 | RT | 1X | 82.65 |
| Ethanol 100% | Various Suppliers | N/A | RT | 70% dilution | Variable |
| PBS-T (0.05% Tween-20) | Prepare fresh | - | 4°C | 0.05% Tween | - |
| RevertAid H Minus RT Buffer (5X) | ThermoFisher Scientific | EP0452 | -20°C | 1X | Included |
| RiboLock RNase Inhibitor (40U/μL) | ThermoFisher Scientific | EO0382 | -20°C | 0.8 U/μL | 262.00 |
| BSA Molecular Biology Grade | NEB | B9000S | -20°C | 0.2 μg/μL | 33.00 |
| dNTP Mix (10mM) | NEB | N0447S | -20°C | 250 μM | 64.00 |
| LNA Primer (oRT_CROPseq-v2-T7-Hybrid) | IDT | Custom | -20°C | 1 μM | 250.00 |
| RevertAid H Minus RT (200U/μL) | ThermoFisher Scientific | EP0452 | -20°C | 4.8 U/μL | 499.00 |
| Glutaraldehyde 10% | Electron Microscopy Sciences | 16120 | 4°C | 0.1% | 32.00 |
| Padlock Probe (oPD_CROPseq-v2-T7-Hybrid) | IDT | Custom | -20°C | 0.1 μM | 200.00 |
| RNase H (5000 U/ml) | Qiagen/Enzymatics | Y9220L | -20°C | 0.4 U/μL | 443.00 |
| Ampligase Enzyme and Buffer | Lucigen/Epicentre | A0102K | -20°C | 0.5 U/μL | 269.00 |
| TaqIT DNA Polymerase (50000 U/mL) | QIAGEN/Enzymatics | P7620L | -20°C | 0.02 U/μL | 514.00 |
| Phi29 DNA Polymerase (10 U/μL) | ThermoFisher Scientific | EP0094 | -20°C | 1 U/μL | 918.00 |
| Phi29 DNA Polymerase Buffer (10X) | ThermoFisher Scientific | B62 | -20°C | 1X | 20.65 |
| Glycerol 100% | Various Suppliers | N/A | RT | 5% v/v | Variable |
| UltraPure DEPC-Treated Water | ThermoFisher Scientific | 750023 | RT | - | 92.50 |
| PCR Film Seals | Various | - | RT | - | Variable |

Note: Refer to OPS_Reagents_Master_List.csv for complete pricing and supplier information.

### 2.3 Software Requirements

Microscope control software (if performing imaging)

Image analysis software (optional)

## 3 Reagent Preparation

### 3.1 4% Paraformaldehyde (PFA)

Final Volume: 50 mL Storage: Prepare fresh, use within 4 hours Shelf Life: Same day use only

| Component | Stock Conc. | Final Conc. | Volume | Notes |
| --- | --- | --- | --- | --- |
| PFA 32% | 32% | 4% | 6.25 mL | Handle in fume hood |
| PBS 1X | 1X | 1X | 43.75 mL | RNase-free |

Preparation Steps:

⚠️ Work in fume hood - PFA is carcinogenic

Add 6.25 mL of 32% PFA to 43.75 mL of 1X PBS

Mix thoroughly and use immediately

### 3.2 Post-fixation Solution

Final Volume: 50 mL Storage: Prepare fresh Shelf Life: Same day use only

| Component | Stock Conc. | Final Conc. | Volume | Notes |
| --- | --- | --- | --- | --- |
| PFA 32% | 32% | 3.2% | 5 mL |  |
| Glutaraldehyde 10% | 10% | 0.1% | 0.5 mL |  |
| PBS 1X | 1X | 1X | 44.5 mL | RNase-free |

### 3.3 Reverse Transcription Mix

Final Volume: 1000 μL (per well of 6-well plate) Storage: Prepare fresh on ice

| Component | Stock Conc. | Final Conc. | Volume | Notes |
| --- | --- | --- | --- | --- |
| 5X RT Buffer | 5X | 1X | 200 μL |  |
| RNase Inhibitor | 40 U/μL | 0.8 U/μL | 20 μL |  |
| BSA | 20 μg/μL | 0.2 μg/μL | 10 μL |  |
| dNTPs | 10 mM | 0.25 mM | 25 μL | Pre-dilute to 10 μM |
| LNA primer | 100 μM | 1 μM | 10 μL |  |
| Reverse Transcriptase | 200 U/μL | 4.8 U/μL | 24 μL | Add last |
| DEPC water | - | - | 711 μL |  |

### 3.4 Ligation Mix

Final Volume: 750 μL (per well of 6-well plate) Storage: Prepare fresh on ice Shelf Life: Same day use only

| Component | Stock Conc. | Final Conc. | Volume per well | Notes |
| --- | --- | --- | --- | --- |
| 10X Ampligase Buffer | 10X | 1X | 75 μL |  |
| Gap filling padlock probes | 2 μM | 0.1 μM | 37.5 μL |  |
| RNase H | 5 U/μL | 0.4 U/μL | 60 μL |  |
| BSA | 20 μg/μL | 0.2 μg/μL | 7.5 μL |  |
| TaqIT/KlenTaq polymerase | 50 U/μL | 0.02 U/μL | 0.3 μL |  |
| dNTPs (pre-diluted) | 10 μM | 50 nM | 3.75 μL | Pre-dilute 1:1,000 from 10mM stock |
| Ampligase | 100 U/μL | 0.5 U/μL | 3.75 μL |  |
| DEPC water | - | - | 562.2 μL |  |

### 3.5 Rolling Circle Amplification (RCA) Mix

Final Volume: 900 μL (per well of 6-well plate) Storage: Prepare fresh on ice Shelf Life: Same day use only

| Component | Stock Conc. | Final Conc. | Volume per well | Notes |
| --- | --- | --- | --- | --- |
| 10X Phi29 Buffer | 10X | 1X | 90 μL |  |
| dNTPs | 10 mM | 0.25 mM | 22.5 μL |  |
| BSA | 20 μg/μL | 0.2 μg/μL | 9 μL |  |
| Glycerol | 50% | 5% v/v | 90 μL |  |
| Phi29 DNA Polymerase | 10 U/μL | 1 U/μL | 90 μL | Add last, temperature sensitive |
| DEPC water | - | - | 598.5 μL |  |

Preparation Notes:

⚠️ Critical Point: Phi29 is temperature sensitive - prepare on ice

Add Phi29 DNA Polymerase last before use

## 4 Procedure

### 4.1 Sample Preparation

Time Required: 2 hours Temperature: Room temperature (RT)

Cell Culture Setup Ensure cells are at 70% confluence and have been in culture for 48 hours ✅ Quality Check: Verify cell health and confluence under microscope

Initial Fixation Remove culture media and add freshly prepared 4% PFA Incubate for 30 minutes at RT ⚠️ Critical Point: Use fresh PFA solution, work in fume hood

Washing Discard formaldehyde according to hazardous waste protocols Wash samples with 1X PBS three times, quick washes ✅ Quality Check: Ensure complete removal of PFA

Permeabilization Add 70% ethanol to cover cells, or any other permeabilization procedure according to the desired phenotype staining protocol (e.g. IF) Incubate for 30 minutes at RT for 70% ethanol, or the appropriate amount of time for alternative permeabilization protocol ⚠️ Critical Point: Avoid cell dehydration if using 70% ethanol

Ethanol Removal (if using 70% EtOH permeabilization) Carefully remove ethanol Add and remove PBS-T (0.05% Tween) 6 times Continue until ethanol concentration is below 1%

PBS Washes Wash with PBS-T three times

### 4.2 Reverse Transcription

Time Required: 6-16 hours Temperature: 37°C

RT Mix Application Apply RT mix to each well ⚠️ Critical Point: Ensure complete coverage of cells

Incubation Fill empty wells and space in between the wells with RNase-free water to minimize evaporation Cover with aluminum foil microseal Incubate at 37°C overnight (minimum 6 hours) 🔧 Troubleshooting: If time is limiting, 6-hour incubation may be sufficient

Post-RT Washing Wash with 1X PBS-T six times, quick washes ⚠️ Critical Point: Complete dNTP removal is essential for subsequent steps

### 4.3 Post-fixation

Time Required: 45 minutes Temperature: Room temperature

Second Fixation Apply post-fixation solution (3.2% PFA + 0.1% Glutaraldehyde) Incubate for 30 minutes at RT ⚠️ Critical Point: This cross-links newly synthesized cDNA

Washing Wash with PBS-T three times, quick washes

### 4.4 mRNA Degradation and Padlock Probe Processing

Time Required: 2 hours Temperature: 37°C then 45°C

Ligation Mix Preparation Prepare ligation mix according to Section 4.4 ⚠️ Critical Point: Pre-dilute dNTPs 1:1,000 from 10mM stock

Thermocycling Apply ligation mix to samples Cover plate and load onto flat thermocycler 5 minutes at 37°C 90 minutes at 45°C

Washing Wash with PBS-T three times, quick washes

### 4.5 Rolling Circle Amplification (RCA)

Time Required: 16 hours Temperature: 30°C

RCA Mix Preparation Prepare RCA mix according to Section 4.5 ⚠️ Critical Point: Phi29 is temperature sensitive - prepare on ice

RCA Incubation Apply RCA mix quickly (Phi29 unstable when diluted) Fill empty wells and space in between the wells with RNase-free water to minimize evaporation Cover plate and incubate at 30°C overnight

Final Washing Wash with PBS-T three times, quick washes

### 4.6 Optional Quality Check

Time Required: 20 minutes Temperature: Room temperature

DNA-FISH Stain sample in DNA-FISH with fluorescently labelled sequencing primer oSBS_CROPseq-v2 (Cy5) to look for Phi29 amplification products (bright punctate signals)

DAPI Application Apply DAPI solution in 1X PBS (1:4000) for 20 minutes Leave wells with 1X PBS until imaging

## 5 Quality Control and Validation

### 5.1 Critical Control Points

CCP 1: Cell confluence at 70% before starting

CCP 2: Complete PFA removal after fixation

CCP 3: Ethanol concentration below 1% after permeabilization

CCP 4: Complete dNTP removal after RT

CCP 5: Visible amplification products after RCA

### 5.2 Expected Results

Cells should remain attached to the plate throughout the procedure

After RCA, bright punctate signals should be visible by fluorescence microscopy

Background fluorescence should be minimal

Cell morphology should be preserved

### 5.3 Optional Validation Steps

Anchor probe hybridization can be performed to visualize amplification products

DAPI staining for nuclei visualization

Phenotyping before or after the procedure

## 6 Troubleshooting

| Problem | Possible Cause | Solution | Prevention |
| --- | --- | --- | --- |
| Cell detachment | Harsh washing | Use gentler pipetting | Optimize wash procedures |
| Low signal | Poor RT efficiency | Check primer design, extend RT time | Verify reagent quality |
| High background | Incomplete washing | Increase wash steps and duration | Follow wash protocols strictly |
| No amplification | Phi29 degradation | Keep enzyme cold, use fresh aliquots | Store properly, minimize freeze-thaw |
| Signal saturation | Over-amplification | Reduce RCA time or enzyme concentration | Optimize conditions for cell type |

## 7 References and Related Documents

### 7.1 Related SOPs

SOP_009b: T7 Library Preparation

SOP_010: SBS Illumina Sequencing

SOP_011: Imaging Using Nikon Ti2

### 7.2 Literature References

[Primary OPS methodology reference]

[Padlock probe design reference]

[Rolling circle amplification optimization reference]

## 8 Appendices

### 8.1 Reagent Volume Calculations

For 6-well plate: multiply volumes by number of wells + 10% excess

For different plate formats: adjust volumes proportionally to surface area

### 8.2 Equipment Settings

Thermocycler: Flat-top, heated lid disabled for this protocol

Incubator: 30°C for RCA, 37°C for RT, humidity control recommended

Microscope: DAPI filter for nuclei, appropriate filters for fluorescent probes